Name: Md. Zahid Hasan, ID: 23-54242-3

clc clear all close all %%%%%%%%%%%%%%%%%%%%%%%%%%%%%%% % Analog to Digital Conversion %%%%%%%%%%%%%%%%%%%%%%%%%%%%%%% time_duration = 0.2; %% Analog-like signal's representation % Analog signal generation is not possible in MATLAB a = [0.4 0.6 0.8]; % amplitude array for composite signal f = [5 12 20]; % frequency array for composite signal analog_t = 0:0.0001:time_duration; analog_sig = a(1)*sin(2*pi*f(1)*analog_t) + a(2)*cos(2*pi*f(2)*analog_t) + a(3)*sin(2*pi*f(3)*analog_t + pi/4); figure subplot(1,2,1) plot(analog_t, analog_sig,'linewidth',1.5) grid on xlabel('time in seconds') ylabel('amplitude in volts') title('analog signal') %% Sampling Frequency fs = 250; ts = 1/fs; %% Sampling samp_t = 0:1/fs:time_duration; samp_sig = a(1)*sin(2*pi*f(1)*samp_t) + a(2)*cos(2*pi*f(2)*samp_t) + a(3)*sin(2*pi*f(3)*samp_t + pi/4); subplot(1,2,2) plot(samp_t, samp_sig,'o','linewidth',1.5) grid on xlabel('time in seconds') ylabel('amplitude in volts') title(['sampled signal for ',num2str(fs),' Hz sampling frequency']) %% Levels for Quantization L = 8; %% Quantizing delta = (max(samp_sig) - min(samp_sig))/(L-1); % step size quant_sig = min(samp_sig) + round((samp_sig-min(samp_sig))/delta)*delta; % quantized signal figure subplot(1,2,1) plot(samp_t, samp_sig,'o','linewidth',1.5) grid on xlabel('time in seconds') ylabel('amplitude in volts') title('sampled signal') subplot(1,2,2) plot(samp_t, quant_sig,'x','linewidth',1.5); xlabel('time') ylabel('amplitude') title('quantized samples') %% Number of Bits/Sample nb = log2(L); %% Encoding i = round((samp_sig-min(samp_sig))/delta); % index for encoding dig_data_matrix = de2bi(i,nb); % encoded binary bits are as a matrix here dig_data = reshape(dig_data_matrix',1,[]); % encoded binary bits are as an array here disp(['The index values for encoding from quantization of the sampled signal are: ',num2str(i)]) disp(['The converted bits from the input analog signal are: ',num2str(dig_data)])

Name: Md. Zahid Hasan, ID: 23-54242-3

Output Figure 1 for Sampling

Output Figure 2 for Quantizing:

Name: Md. Zahid Hasan, ID: 23-54242-3

Performance Task for Lab Report: (your ID = 23-54242-3)

MATLAB Code:

clc clear all close all % Analog to Digital Conversion time_duration = 0.2; %% Values of F and G (change according to your ID) F = 3; G = 4; %% Amplitude and Frequency according to question a1 = F + 1; a2 = F + 3; a3 = F + 2; a4 = F + 4; f1 = G + 5; f2 = G + 7; f3 = G + 1; f4 = G + 2; %% Analog Signal analog_t = 0:0.0001:time_duration; sig = a1*sin(2*pi*f1*analog_t) + ... a2*cos(2*pi*f2*analog_t) + ... a3*sin(2*pi*f3*analog_t) + ... a4*sin(2*pi*f4*analog_t); figure subplot(1,2,1) plot(analog_t, sig,'linewidth',1.5) grid on xlabel('time (seconds)') ylabel('amplitude') title('Analog Signal') %% Sampling Frequency fs = 250; % sampling frequency ts = 1/fs; %% Sampling samp_t = 0:ts:time_duration; samp_sig = a1*sin(2*pi*f1*samp_t) + ... a2*cos(2*pi*f2*samp_t) + ... a3*sin(2*pi*f3*samp_t) + ... a4*sin(2*pi*f4*samp_t); subplot(1,2,2) stem(samp_t, samp_sig,'filled') grid on xlabel('time (seconds)') ylabel('amplitude') title(['Sampled Signal (fs = ',num2str(fs),' Hz)']) %% Quantization Levels L = 32; % number of levels %% Quantization delta = (max(samp_sig) - min(samp_sig))/(L-1); quant_sig = min(samp_sig) + round((samp_sig-min(samp_sig))/delta)*delta; figure subplot(1,2,1) stem(samp_t, samp_sig,'filled') grid on xlabel('time') ylabel('amplitude') title('Sampled Signal') subplot(1,2,2) stem(samp_t, quant_sig,'r') grid on xlabel('time') ylabel('amplitude') title('Quantized Signal') %% Number of Bits nb = log2(L); %% Encoding (Digital Data) i = round((samp_sig-min(samp_sig))/delta); dig_data_matrix = de2bi(i,nb); dig_data = reshape(dig_data_matrix',1,[]); disp('Index values from quantization:') disp(i) disp('Digital bit stream:') disp(dig_data)

Name: Md. Zahid Hasan, ID: 23-54242-3

a) Show analog signal, sampled signal, and quantized signal.

Name: Md. Zahid Hasan, ID: 23-54242-3

b) Show the digital data from the analog signal.

Index values from quantization:

Columns 1 through 31

23    26    28    30    31    31    31    30    29    28    27    26    25    24    24    23    23    22    22    22    21    20    19    18    16    14    12     9     7     5     3

Columns 32 through 51

2     1     0     0     1     2     3     5     8    10    13    15    17    19    20    21    22    22    22    22

Digital bit stream:

Columns 1 through 31

1     1     1     0     1     0     1     0     1     1     0     0     1     1     1     0     1     1     1     1     1     1     1     1     1     1     1     1     1     1     1

Columns 32 through 62

1     1     1     1     0     1     1     1     1     1     0     1     1     1     0     0     1     1     1     1     1     0     1     1     0     1     0     1     1     1     0

Columns 63 through 93

0     1     1     0     0     0     1     1     0     0     0     1     1     1     1     1     0     1     1     1     1     0     1     0     1     1     0     1     0     1     1

Columns 94 through 124

0     1     0     1     1     0     1     1     0     1     0     1     0     0     1     0     1     1     1     0     0     1     0     1     0     0     1     0     0     0     0

Columns 125 through 155

1     0     1     1     1     0     0     0     1     1     0     1     0     0     1     0     1     1     1     0     0     1     0     1     0     0     1     1     0     0     0

Columns 156 through 186

0     1     0     0     0     1     0     0     0     0     0     0     0     0     0     0     0     0     0     0     1     0     0     0     0     0     1     0     0     0     1

Columns 187 through 217

1     0     0     0     1     0     1     0     0     0     0     0     1     0     0     1     0     1     0     1     0     1     1     0     1     1     1     1     0     1     0

Columns 218 through 248

0     0     1     1     1     0     0     1     0     0     1     0     1     1     0     1     0     1     0     1     1     0     1     0     1     1     0     1     0     1     1

Columns 249 through 255

0     1     0     1     1     0     1

Name: Md. Zahid Hasan, ID: 23-54242-3

c) What are the appropriate values of sampling frequency and number of levels of quantization if minimum required SNR and bandwidth of the channel are 25 dB and 150 Hz respectively.