Name: MD. Zahid Hasan

Id: 23-54242-3

Task:1

MATLAB Code: Unipolar NRZ

clc

clear all

close all

bit_stream = [1 0 0 0 1 1 0 1];

no_bits = length(bit_stream);

bit_rate = 1000; % 1 kbps

pulse_per_bit = 1; % for unipolar nrz

pulse_duration = 1/((pulse_per_bit)*(bit_rate));

no_pulses = no_bits*pulse_per_bit;

samples_per_pulse = 500;

fs = (samples_per_pulse)/(pulse_duration); %sampling frequency

% including pulse duration in sampling frequency

% ensures having enough samples in each pulse

t = 0:1/fs:(no_pulses)*(pulse_duration); % sampling interval

% total duration = (no_pulse)*(pulse_duration)

no_samples = length(t); % total number of samples

dig_sig = zeros(1,no_samples);

max_voltage = 5;

min_voltage = 0;

for i = 1:no_bits

if bit_stream(i) == 1

dig_sig(((i-1)*(samples_per_pulse)+1):i*(samples_per_pulse)) = max_voltage*ones(1,samples_per_pulse);

else

dig_sig(((i-1)*(samples_per_pulse)+1):i*(samples_per_pulse)) = min_voltage*ones(1,samples_per_pulse);

end

end

plot(t,dig_sig,'linewidth',1.5)
grid on

xlabel('time in seconds')

ylabel('Voltage')

ylim([(min_voltage - (max_voltage)*0.2) (max_voltage+max_voltage*0.2)])

title(['Unipolar NRZ for ',num2str(bit_stream),''])

Task 2. MATLAB Code Unipolar RZ:

clc clear all close all bit_stream = [1 0 1 0 1 1 0 1]; no_bits = length(bit_stream); bit_rate = 1000; % 1 kbps pulse_per_bit = 2; % for unipolar rz pulse_duration = 1/((pulse_per_bit)*(bit_rate)); no_pulses = no_bits*pulse_per_bit; samples_per_pulse = 500; fs = (samples_per_pulse)/(pulse_duration); %sampling frequency % including pulse duration in sampling frequency % ensures having enough samples in each pulse t = 0:1/fs:(no_pulses)*(pulse_duration); % sampling interval % total duration = (no_pulse)*(pulse_duration) no_samples = length(t); % total number of samples dig_sig = zeros(1,no_samples); max_voltage = 4; min_voltage = 0; for i = 1:no_bits j = (i-1)*2; if bit_stream(i) == 1 dig_sig((j*(samples_per_pulse)+1):(j+1)*(samples_per_pulse)) = max_voltage*ones(1,samples_per_pulse); dig_sig(((j+1)*(samples_per_pulse)+1):(j+2)*(samples_per_pulse)) = zeros(1,samples_per_pulse); else dig_sig((j*(samples_per_pulse)+1):(j+1)*(samples_per_pulse)) = min_voltage*ones(1,samples_per_pulse); dig_sig(((j+1)*(samples_per_pulse)+1):(j+2)*(samples_per_pulse)) = zeros(1,samples_per_pulse); end end plot(t,dig_sig,'linewidth',1.5) grid on xlabel('time in seconds') ylabel('Voltage') ylim([(min_voltage - (max_voltage)*0.2) (max_voltage+max_voltage*0.2)]) title(['Unipolar RZ for ',num2str(bit_stream),''])

Name: MD. Zahid Hasan

Id: 23-54242-3

Task 3. MATLAB code for Differential Manchester:

Clc clear all close all bit_stream = [1 1 0 0 1 1]; no_bits = length(bit_stream); bit_rate = 1000; % 1 kbps pulse_per_bit = 2; % for differential manchester pulse_duration = 1/((pulse_per_bit)*(bit_rate)); no_pulses = no_bits*pulse_per_bit; samples_per_pulse = 500; fs = (samples_per_pulse)/(pulse_duration); %sampling frequency % including pulse duration in sampling frequency % ensures having enough samples in each pulse t = 0:1/fs:(no_pulses)*(pulse_duration); % sampling interval % total duration = (no_pulse)*(pulse_duration) no_samples = length(t); % total number of samples dig_sig = zeros(1,no_samples); max_voltage = +2; min_voltage = -2; inv_bit = 1; % inverting bit last_state = max_voltage; inv_last_state = min_voltage; % inverse of last state for i = 1:no_bits j = (i-1)*2; if bit_stream(i) == inv_bit dig_sig((j*(samples_per_pulse)+1):(j+1)*(samples_per_pulse)) = inv_last_state*ones(1,samples_per_pulse); dig_sig(((j+1)*(samples_per_pulse)+1):(j+2)*(samples_per_pulse)) = last_state*ones(1,samples_per_pulse); else dig_sig((j*(samples_per_pulse)+1):(j+1)*(samples_per_pulse)) = last_state*ones(1,samples_per_pulse); dig_sig(((j+1)*(samples_per_pulse)+1):(j+2)*(samples_per_pulse)) = inv_last_state*ones(1,samples_per_pulse); temp_cons = last_state; % temporary constant last_state = inv_last_state; inv_last_state = temp_cons; end end figure plot(t,dig_sig,'linewidth',1.5) grid on xlabel('time in seconds') ylabel('Voltage') ylim([(min_voltage - (max_voltage)*0.2) (max_voltage+max_voltage*0.2)]) title(['Differential Manchester for ',num2str(bit_stream),', last state = ',num2str(last_state),', inverting bit is ',num2str(inv_bit),''])

Name: MD. Zahid Hasan

Id: 23-54242-3

Performance Task:

- Polar NRZ-L (4 kbps)

clc;

clear;

close all;

bit_stream = [0 0 1 0 0 1 0 0 0 0 1 0];

Rb = 4000;                 % 4 kbps

Tb = 1/Rb;

samples = 200;

t = 0:Tb/samples:Tb*length(bit_stream);

signal = [];

for i = 1:length(bit_stream)

if bit_stream(i)==1

signal = [signal ones(1,samples)];

else

signal = [signal -ones(1,samples)];

end

end

t = linspace(0,Tb*length(bit_stream),length(signal));

plot(t,signal,'LineWidth',1.5)

ylim([-1.5 1.5])

grid on

title('Polar NRZ-L (4 kbps)')

xlabel('Time (s)')

ylabel('Amplitude')

- Manchester (2 kbps)

clc;

clear;

close all;

bit_stream = [0 0 1 0 0 1 0 0 0 0 1 0];

Rb = 2000;           % 2 kbps

Tb = 1/Rb;

samples = 200;

signal = [];

for i = 1:length(bit_stream)

if bit_stream(i)==1

signal = [signal ones(1,samples/2) -ones(1,samples/2)];

else

signal = [signal -ones(1,samples/2) ones(1,samples/2)];

end

end

t = linspace(0,Tb*length(bit_stream),length(signal));

plot(t,signal,'LineWidth',1.5)

ylim([-1.5 1.5])

grid on

title('Manchester (2 kbps)')

xlabel('Time (s)')

ylabel('Amplitude')

Name: MD. Zahid Hasan

Id: 23-54242-3

- AMI (5 kbps)

clc;

clear;

close all;

bit_stream = [0 0 1 0 0 1 0 0 0 0 1 0];

Rb = 5000;

Tb = 1/Rb;

samples = 200;

signal = [];

polarity = 1;

for i = 1:length(bit_stream)

if bit_stream(i)==1

signal = [signal polarity*ones(1,samples)];

polarity = -polarity;

else

signal = [signal zeros(1,samples)];

end

end

t = linspace(0,Tb*length(bit_stream),length(signal));

plot(t,signal,'LineWidth',1.5)

ylim([-1.5 1.5])

grid on

title('AMI (5 kbps)')

xlabel('Time (s)')

ylabel('Amplitude')

- MLT-3 (10 kbps)

clc;

clear;

close all;

bit_stream = [0 0 1 0 0 1 0 0 0 0 1 0];

Rb = 10000;

Tb = 1/Rb;

samples = 200;

signal = [];

levels = [0 1 0 -1];

index = 1;

for i = 1:length(bit_stream)

if bit_stream(i)==1

index = index + 1;

if index > 4

index = 1;

end

end

signal = [signal levels(index)*ones(1,samples)];

end

t = linspace(0,Tb*length(bit_stream),length(signal));

plot(t,signal,'LineWidth',1.5)

ylim([-1.5 1.5])

grid on

title('MLT-3 (10 kbps)')

xlabel('Time (s)')

ylabel('Amplitude')