Write a MATLAB program to perform the sequential tasks (a)–(f) outlined below

(a) Generate the composite analog signal sig over the given time vector analog_t using:

f_analog = 1000;

analog_t = 0:1/f_analog : 1-1/f_analog;

sig = 60*sin(2*pi*3*analog_t) + 30*cos(2*pi*8*analog_t) + 20*sin(2*pi*16*analog_t);

(b) Use MATLAB’s built-in functions “fft” and “fftshift” to compute and visualize the frequency-domain representation of the signal sig

(c) Sample the analog signal using the sampling frequency fs and sampling time vector samp_t defined as:

fs = 150;

samp_t = 0:1/fs : 1-1/fs;

samp_sig = 60*sin(2*pi*3*samp_t) + 30*cos(2*pi*8*samp_t) + 20*sin(2*pi*16*samp_t);

(d) Quantize the sampled signal samp_sig using a 4-bit ADC to obtain the quantized signal quant_sig

(e) Using a 2×2 subplot layout, plot:

1.	The original analog signal sig in the time domain

2.	Its frequency-domain spectrum

3.	The sampled signal samp_sig in the time domain

4.	The quantized signal quant_sig in the time domain

(f) Generate the binary encoded codewords corresponding to the quantized signal for synchronous serial transmission, and display the resulting bitstream in the MATLAB command window.

clc;

clear;

close all;

%% (a) Generate Composite Analog Signal

f_analog = 1000;

analog_t = 0:1/f_analog : 1-1/f_analog;

sig = 60*sin(2*pi*3*analog_t) + ...

30*cos(2*pi*8*analog_t) + ...

20*sin(2*pi*16*analog_t);

%% (b) Frequency-Domain Representation using FFT

N = length(sig);

SIG = fft(sig);

SIG_shift = fftshift(SIG);

freq_axis = (-N/2:N/2-1)*(f_analog/N);

%% (c) Sampling the Analog Signal

fs = 150;

samp_t = 0:1/fs : 1-1/fs;

samp_sig = 60*sin(2*pi*3*samp_t) + ...

30*cos(2*pi*8*samp_t) + ...

20*sin(2*pi*16*samp_t);

%% (d) 4-bit Quantization

nbits = 4;

L = 2^nbits;              % Number of quantization levels

xmin = min(samp_sig);

xmax = max(samp_sig);

delta = (xmax - xmin)/L;  % Step size

% Quantization

quant_index = floor((samp_sig - xmin)/delta);

quant_index(quant_index == L) = L-1;   % Prevent overflow

quant_sig = xmin + quant_index*delta + delta/2;

%% (e) 2x2 Subplots

figure;

% 1. Original Analog Signal

subplot(2,2,1);

plot(analog_t, sig);

title('Analog Signal (Time Domain)');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

% 2. Frequency Spectrum

subplot(2,2,2);

plot(freq_axis, abs(SIG_shift)/N);

title('Frequency Spectrum');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

grid on;

% 3. Sampled Signal

subplot(2,2,3);

stem(samp_t, samp_sig, 'filled');

title('Sampled Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

% 4. Quantized Signal

subplot(2,2,4);

stem(samp_t, quant_sig, 'filled');

title('Quantized Signal (4-bit)');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

%% (f) Binary Encoding for Serial Transmission

% Convert quantization indices to 4-bit binary

binary_code = dec2bin(quant_index, nbits);

% Convert matrix of bits into a single serial bitstream

bitstream = reshape(binary_code.', 1, []);

disp('Binary Encoded Bitstream:');

disp(bitstream);